Practical 1 : Packet Tracer - Configure Cisco Routers for Syslog, NTP, and SSH Operations

- Connections

- Server  to Switch1

- PC-C to Switch2

- Switch1 to R1(0/1)

- Switch2 to R3(0/1)

- R1 – Physical – zoom in – power off – HWIC2-T – then drag on black space (both sides) – power on [Same for R2 and R3]

- Connection of routers (using serial DCE wire)

- R1(Serial 0/0/0) – R2 (Serial 0/0/0)

- R2(0/0/1) – R3(0/0/0)

- IP address for PCs

PC – Desktop – IP config

PC-A: ip- 192.168.1.5

Subnet: 255.255.255.0

Default: 192.168.1.1

PC-B: ip: 192.168.1.6

Subnet: 255.255.255.0

Default: 192.168.1.1

PC-C: ip: 192.168.3.5

Subnet: 255.255.255.0

Default: 192.168.3.1

- IP addresses for routers

Router – config – right corner click ON it

R1: G0/1

Ip : 192.168.1.1

Subnet: 255.255.255.0

S0/0/0

Ip: 10.1.1.1

Subnet: 255.255.255.252

R2: S0/0/0

Ip: 10.1.1.2

Subnet: 255.255.255.252

S0/0/1

Ip:10.2.2.2

Subnet: 255.255.255.252

R3: G0/1

Ip: 192.168.3.1

Subnet: 255.255.255.30

S0/0/0

Ip: 10.2.2.1

Subnet: 255.255.255.252

- Check connection

- Ping from PC-A to PC-B

- R1 to R2 , R2 to R3

- R3 to PC-C

Part 1: Configure OSPF MD5 Authentication

- R1 to CLI

ex (pura exit karo)

en

config

router ospf 1

network 192.168.1.0 0.0.0.255 area 0

network 10.1.1.0 0.0.0.3 area 0

- R2 to CLI

ex

en

config

router ospf 2

network 10.1.1.0 0.0.0.3 area 0

network 10.2.2.0 0.0.0.3 area 0

- R3 to CLI

ex

en

config

router ospf 3

network 192.168.3.0 0.0.0.255 area 0

network 10.2.2.0 0.0.0.3 area 0

[message will show as Loading data]

- Now connection from

PC to router

- PC-A – Desktop – Command Prompt

- ping 192.168.3.5

PC-B

- ping 192.168.1.6

- Router CLI

R1:

router ospf 1

area 0 authentication message-digest

R2:

router ospf 2

area 0 authentication message-digest

R3:

router ospf 3

area 0 authentication message-digest

- Again CLI

R1:

interface s0/0/0

ip ospf message-digest-key 1 md5 MD5pa

R2:

interface s0/0/0

ip ospf message-digest-key 1 md5 MD5pa

interface s0/0/1

ip ospf message-digest-key 1 md5 MD5pa

R3:

interface s0/0/1

ip ospf message-digest-key 1 md5 MD5pa

- Again CLI

R1:

ex (exit till it show # )

show ospf interface [output in paragraph]

Part 2 : Configure NTP

- PC-A – Services – NTP

- Click enable

- Key-1, password:NTPpa55

- CLI

- ntp server 192.168.1.5   (same for all servers)

- Again CLI

- ntp update – calendar  (same for all servers)

- CLI   (same for all routers)

ntp authenticate

ntp trusted – key 1

ntp auithentication – key 1 mds NTPpa55

- CLI   (same for all routers)

service timestamps log datetime msec

[to check: router – CLI – ex – show]

Practical 2: Packet Tracer - Configure AAA Authentication on Cisco Routers

Topology

- Connection

- PC-B to S2

- TACACS+Server to S2

- PC-A to S1

- S1 to R1(0/1)

- RADIUS+Server to S3

- PC-C to S3

- S2 to R2(0/0)

- S3 to R3(0/1)

- R1 – Physical – zoom in – power off – HWIC2-T – then drag on black space (both sides) – power on [for all routers]

- Connection of routers (using serial DCE wire)

- R1(Serial 0/0/0) – R2 (Serial 0/0/0)

- R2(0/0/1) – R3(0/0/1)

- IP address given in table

- Check Connection

- Pc to router

- R1 – CLI

- en

- line vty 0 4       (should be in config)

- password admin

- login

- en

- router ospf 1

- network 192.168.1.0 0.0.0.255 area 0

- network 10.1.1.0 0.0.0.255 area 0

R2 – CLI

- en

- line vty 0 4       (should be in config)

- password admin

- login

- en

- router ospf

- network 192.168.2.0 0.0.0.255 area 0

- network 10.2.2.0 0.0.0.255 area 0

- network 10.1.1.0 0.0.0.255 area 0

R3 – CLI

- en

- line vty 0 4       (should be in config)

- password admin

- login

- en

- router ospf 3

- network 192.168.3.0 0.0.0.255 area 0

- network 10.2.2.0 0.0.0.255 area 0

- Check Connection

- Server to server

- Pc to server

- Pc to pc

Part 1: Configure Local AAA Authentication for Console Access on R1

- R1 – CLI

- en

- config

- username Admin1 secret admin1pa55

- aaa new-model

- aaa authentication lofin default local

- line console 0

- login authentication default

- end

- ex

- Username: Admin1

- Password: admin1pa55

[ if goes back to router that is router > , then done]

- Connecting VTY lines to R1(R1 – CLI)

- en

- config

- ip domain-name ccnasecurity.com

- crypto key generate rsa

- aaa authentication login SSH-LOGIN local

- line vty 0 4

- login authentication SSH-LOGIN

- transport input ssh

- end

- ex

- username: Admin1

- password: admin1pa55

- check secure connection of ssh to PC – A

- PC-A

- Desktop

- Command Prompt

- ssh –l Admin1 192.168.1.1

- will ask for password : admin1pa55

Part 3: Configure Server-Based AAA Authentication Using TACACS+ on R2

- R2 – CLI

- en

- config

- username Admin2 secret adminpa55

- TACACS Server – Services – AAA   (ON it)

- Client name: R2

- Client IP: 192.168.2.1

- Secret: tacacspa55

- Server type:Tacacs  (ADD)

- Username: Admin2

- Password: admin2pa55(add)

- R2 – CLI

- aaa new- model

- aaa authentication login default group tacacs+server

- line console 0

- login authentication default

- end

- ex

- Username: Admin2

- Password: admin2pa55

Part 4: Configure Server-Based AAA Authentication Using RADIUS on R3

- R3 – CLI

- en

- config

- username Admin3 secret adminpa55

- RADIUS Server – Services – AAA   (ON it)

- Client name: R3

- Client IP: 192.168.3.1

- Secret: radiuspa55

- Server type:Radius  (ADD)

- Username: Admin3

- Password: admin3pa55(add)

- R3 – CLI

- aaa new- model

- aaa authentication login default group radius+server

- line console 0

- login authentication default

- end

- ex

- Username: Admin3

- Password: admin3pa55

Practical 3: Configuring Extended ACLs - Scenario 1

Topology:

- Connection

- PC1 to S1

- PC2 to C2

- S1 to Router(0/0)

- S2 to Router(0/1)

- Server to S3

- S3 to Router(0/2)

- Router – config

G0/0:

Ip: 172.22.34.65

Subnet: 255.255.255.224

G0/1:

Ip: 172.22.34.97

Subnet: : 255.255.255.240

G0/2:

Ip: 172.22.34.1

Subnet: 255.255.255.192

- Server – Desktop ip config

Ip: 172.22.34.62

Subnet: 255.255.255.192

Default: 172.22.34.1

- IP addresses for PCs

PC1: Ip: 172.22.34.66

Subnet: 255.255.255.224

Default: 172.22.34.65

PC2: Ip: 172.22.34.98

Subnet: 255.255.255.240

Default: 172.22.34.97

- Connection (ping from)

- Server to router

- Router to pc

- Router – CLI

- ex

- en

- config

- access-list 100 permit tcp 172.22.34.64 0.0.0.31 host 172.22.34.62 eq ftp

- access-list 100 permit icmp 172.22.34.64 0.0.0.31 host 172.22.34.62

- interface gigabitethernet 0/0

- ip access-group 100 in

- check connection

PC1 to server

- PC0 – to desktop – command prompt

- ftp 172.22.34.62

- Username: cisco

- Password: cisco

- CLI

- ex

- en

- config

- ip access-list extended HTTP_ONLY

- permit tcp 172.22.34.96 0.0.0.15 host 172.22.34.62 eq www

- interface gigabitEhternet 0/1

- ip access-group HTTP_ONLY in

- PC2 – desktop – web browser

- url: 172.22.34.62

Practical 3: Configuring Extended ACLs - Scenario 2

Topology

- Connection

- PC-A to SWA

- PC-B to SWB

- SWA to Router(0/0)

- SWB to Router(0/1)

- SWC to Router(0/2)

- IP addresses for routers

- G0/0

Ip: 10.101.117.49

Subnet: 255.255.255.248

- G0/1

Ip: 10.101.117.33

Subnet: 255.255.255.240

- G0/2

Ip: 10.101.117.1

Subnet: 255.255.255.224

- IP addresses for PC

- PC-A

Ip: 10.101.117.51

Subnet: 255.255.255.248

Default: 10.101.117.49

- PC-B

Ip: 10.101.117.35

Subnet: 255.255.255.240

Default: 10.101.117.33

- Switch -  CLI

- SWA:

ex

en

config

int Vlan1

ip address 10.101.117.50 255.255.255.248

no shutdown

ip default-gateway 10.101.117.49

- SWB:

ex

en

config

int Vlan1

ip address 10.101.117.34 255.255.255.240

no shutdown

ip default-gateway 10.101.117.33

- SWC:

ex

en

config

int Vlan1

ip address 10.101.117.2 255.255.255.224

no shutdown

ip default-gateway 10.101.117.1

- Switch – CLI (for all switchs)

ex

en

config

enable

enable Secret enpa55

line console 0

Password Conpa55

login

- Switch – CLI  (All switches)

- Ip domain – name ccnaSecurity.com

- username Admin Secret Adminpa55

- line vtg 0 4

- login local

- crypto key generate rsa

[After entering thse commands in all switches go to switch 1 then ex, it will ask password]

- password: Conpa55 (enter)

- en

- password: enpa55 (enter) – this will show Switch #

- can even ask for bits before password so enter 1024

- Router – CLI

- access-list 199 permit tcp 10.101.117.35 0.0.0.15 10.101.117.0 0.0.0.31 eq 22

- access-list 199 permit icmp any any

- int gigabitEthernet 0/2

- ip access-group 199 out

Practical 4: Configure and Verify a Site-to-Site IPsec VPN Using CLI

- Topology

- PC-A to S1

- PC-B to S2

- PC-C to S3

- S1 to R1(0/1)

- S2 to R2(0/0)

- S3 to R3(0/1)

- R1(0/0/0) to R2(0/0/0)

- R2(0/0/1) to R3(0/0/0)

- R1 – Physical – zoom in – power off – HWIC2-T – then drag on black space (both sides) – power on [Same for R2 and R3]

- Connection of routers (using serial DCE wire)

- R1(Serial 0/0/0) – R2 (Serial 0/0/0)

- R2(0/0/1) – R3(0/0/0)

- IP addresss for PCs

PC – Desktop – IP config

PC-A: ip- 192.168.1.3

Subnet: 255.255.255.0

Default: 192.168.1.1

PC-B: ip: 192.168.2.3

Subnet: 255.255.255.0

Default: 192.168.2.1

PC-C: ip: 192.168.3.3

Subnet: 255.255.255.0

Default: 192.168.3.1

- IP addresses for routers

Router – config – right corner click ON it

R1: G0/1

Ip : 192.168.1.1

Subnet: 255.255.255.0

S0/0/0

Ip: 10.1.1.2

Subnet: 255.255.255.252

R2: S0/0/0

Ip: 10.1.1.1

Subnet: 255.255.255.252

S0/0/1

Ip:10.2.2.1

Subnet: 255.255.255.252

R3: G0/1

Ip: 192.168.3.1

Subnet: 255.255.255.30

S0/0/0

Ip: 10.2.2.2

Subnet: 255.255.255.252

- Router – CLI

R1:

ex

en

config

router ospf 1

network 192.168.1.0 0.0.0.255 area 0

network 10.1.1.0 0.0.0.3 area 0

R2:

ex

en

config

router ospf 2

network 10.1.1.0 0.0.0.3 area 0

network 192.168.2.0 0.0.0.255 area 0

R3:

ex

en

config

router ospf 3

network 192.168.3.0 0.0.0.255 area 0

network 10.2.2.0 0.0.0.3 area 0

- Check connection

- PC to PC

- Router – CLI  (R1 and R3)

- license boot module c1900 technology-package securityk9

- [Yes/no]: Y

- ex

- router# reload

- [Yes/no]: Y

- [confirm]: Y

- R1 – CLI

- ex

- en

- config

- access-list 110 permit ip 192.168.1.0 0.0.0.255 192.168.3.0 0.0.0.255

- crypto isakmp policy 10

- encryption aes 256

- authentication pre-share

- group 5

- exit

- crypto isakmp key vpnpa55 address 10.2.2.2

- crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac

- crypto map VPN-MAP 10 ipsec-isakmp

- description VPN connection to R3

- set peer 10.2.2.2

- set transform-set VPN-SET

- match address 110

- exit

- interface s0/0/0

- crypto map VPN-MAP

- R3 – CLI

- ex

- en

- config

- access-list 110 permit ip 192.168.3.0 0.0.0.255 192.168.2.0 0.0.0.255

- crypto isakmp policy 10

- encryption aes 256

- authentication pre-share

- group 5

- exit

- crypto isakmp key vpnpa55 address 10.1.1.2

- crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac

- crypto map VPN-MAP 10 ipsec-isakmp

- description VPN connection to R3

- set peer 10.1.1.2

- set transform-set VPN-SET

- match address 110

- exit

- interface s0/0/1

- crypto map VPN-MAP

[ this will show ISAKMP is ON]